For combinations cells (depending on number of inputs – example, inverter has only A input), the following characterizations have to be performed and filled. Remove all unwanted rows.

- Input pin capacitances:

| Input Pins | Rise Cap (fF) | Fall Cap (fF) | Average Cap (fF) |
| --- | --- | --- | --- |
| A | 3.86 | 3.95 | 3.91 |
| B | 4.42 | 4.40 | 4.41 |
| C | 4.175 | 4.173 | 4.174 |

- Transition Time Table: (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 25.89 | 28.32 | 26.49 |
| 10 fF | 72.04 | 72.22 | 76.19 |
| 100 fF | 515.16 | 515.17 | 643.66 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 32.98 | 33.92 | 33.92 |
| 10 fF | 79.81 | 86.68 | 144.9 |
| 100 fF | 522.83 | 510.57 | 497.76 |

Related pin C: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 31.27 | 34.95 | 31.89 |
| 10 fF | 51.73 | 53.9 | 54.67 |
| 100 fF | 381 | 381 | 376.33 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 41.11 | 40.46 | 108.65 |
| 10 fF | 86.66 | 86.36 | 170.17 |
| 100 fF | 510.58 | 510.60 | 521.65 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 41.44 | 40.79 | 40.79 |
| 10 fF | 86.81 | 86.68 | 144.9 |
| 100 fF | 510.6 | 510.57 | 521.32 |

Related pin C: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 37.47 | 40.9 | 141.35 |
| 10 fF | 79.5 | 79.7 | 177 |
| 100 fF | 612.2 | 612.2 | 643.43 |

- Propagation delay time tables: (unlike textbook definitions that we used for our assignments, here we will use 50% of input to 50% of output to simulate propagation delay – by keeping other inputs fixed).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 1046.98 | 1156.46 | 1535.3 |
| 10 fF | 1080.9 | 1191.40 | 1569.7 |
| 100 fF | 1366.55 | 1485.6 | 1533.88 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 1040.43 | 1139.87 | 1543.99 |
| 10 fF | 1030.33 | 1167.22 | 1548.33 |
| 100 fF | 1266.34 | 1365.87 | 1534.99 |

Related pin C: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 1037.6 | 1145.7 | 1526.7 |
| 10 fF | 1050.6 | 1160 | 1540.35 |
| 100 fF | 1238.5 | 1357.7 | 1585.22 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 50.31 | 66.27 | 168.22 |
| 10 fF | 88.4 | 105.03 | 242.13 |
| 100 fF | 413.77 | 431.06 | 365.87 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 40.55 | 50.32 | 143.77 |
| 10 fF | 80.76 | 100.54 | 238.76 |
| 100 fF | 450.54 | 493.22 | 400.53 |

Related pin C: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 33.2 | 49.44 | 123.68 |
| 10 fF | 77.36 | 94.45 | 231.78 |
|  | 486.37 | 507.75 | 654.99 |

- Static Power (cover all possible input combinations based on number of inputs).

| Condition (ABC) | Power (nW) |
| --- | --- |
| 000 | 131 |
| 001 | 1.72 |
| 010 | 54.4 |
| 011 | 65.8 |
| 100 | 4.9 |
| 101 | 1.72 |
| 110 | 0.05 |
| 111 | 0.05 |

- Dynamic Power Table:

(i) Rise Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 12171 | 11763 | 12414 |
| 10 fF | 27749 | 27439 | 27504 |
| 100 fF | 164833 | 163974 | 153631 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 14073 | 13812 | 14277 |
| 10 fF | 29483 | 29306 | 29461 |
| 100 fF | 166182 | 165269 | 154771 |

Related pin C: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 8356 | 8149 | 10854 |
| 10 fF | 23911 | 23723 | 25322 |
| 100 fF | 168004 | 167765 | 163504 |

(ii) Fall Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 2800 | 2050 | 2472 |
| 10 fF | 2856 | 2268 | 2469 |
| 100 fF | 1868 | 1485 | 1099 |

Related pin B: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 2870 | 2162 | 2374 |
| 10 fF | 2832 | 2285 | 2153 |
| 100 fF | 1776 | 1402 | 982 |

Related pin C: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 1809 | 2625 | 945 |
| 10 fF | 1486 | 2149 | 2012 |
| 100 fF | 1382 | 1393 | 2194 |