For Sequential cells, the following characterizations have to be performed and filled.

- Input pin capacitances:

| Input Pins | Rise Cap (fF) | Fall Cap (fF) | Average Cap (fF) |
| --- | --- | --- | --- |
| D | 4.01855 | 4.01825 | 4.0184 |
| CLK | 20.9715 | 20.9719 | 20.9717 |

- Set-up Time Table:

(i) Rise Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0.152 | 0.333 |
| 1000 ps | 0.243 | 0.409 |

(ii) Fall Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0.158 | 0.355 |
| 1000 ps | 0 | 0.17 |

- Hold Time Table:

(i) Rise Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0 | 0 |
| 1000 ps | 0.034 | 0 |

(ii) Fall Constraint (in ns) [Input slew vs CLK slew].

|  | 10 ps | 1000 ps |
| --- | --- | --- |
| 10 ps | 0 | 0 |
| 1000 ps | 0 | 0 |

- Transition Time Table (for Q output, CLK will have minimum slew of 10 ps): (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.018 | 0.018 | 0.018 |
| 10 fF | 0.068 | 0.068 | 0.068 |
| 100 fF | 0.596 | 0.596 | 0.596 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.027 | 0.027 | 0.027 |
| 10 fF | 0.083 | 0.083 | 0.083 |
| 100 fF | 0.644 | 0.644 | 0.644 |

- CLK-to-Q Delay Time Table: (delay between clock transition and data transition. Use 50% of CLK to 50% of output to simulate propagation delay).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.196 | 0.196 | 0.196 |
| 10 fF | 0.244 | 0.244 | 0.244 |
| 100 fF | 0.596 | 0.596 | 0.596 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.288 | 0.288 | 0.288 |
| 10 fF | 0.346 | 0.346 | 0.346 |
| 100 fF | 0.785 | 0.785 | 0.785 |

- Static Power (all possible input combinations of CLK and D).

| Condition (CLK, D) | Power (nW) |
| --- | --- |
| 00 | 4.226 |
| 01 | 5.252 |
| 10 | 0.429 |
| 11 | 0.612 |

- Dynamic Power Table: (CLK will have minimum slew of 10 ps)

(i) Rise Power (in nW) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 37195.4 | 37038.6 | 37098.6 |
| 10 fF | 44971.4 | 44814.6 | 44870.9 |
| 100 fF | 117642 | 117485 | 117541 |

(ii) Fall Power (in nW) [Input slew vs output capacitance].

Related pin D: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 42921.7 | 42821.2 | 42862.8 |
| 10 fF | 43065.5 | 42965.1 | 43007.3 |
| 100 fF | 43197.4 | 43097 | 43137.7 |