For combinations cells (depending on number of inputs – example, inverter has only A input), the following characterizations have to be performed and filled. Remove all unwanted rows.

- Input pin capacitances:

| Input Pins | Rise Cap (fF) | Fall Cap (fF) | Average Cap (fF) |
| --- | --- | --- | --- |
| A | 2.915 | 2.915 | 2.915 |

- Transition Time Table: (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.014 | 0.023 | 0.084 |
| 10 fF | 0.066 | 0.068 | 0.173 |
| 100 fF | 0.577 | 0.577 | 0.588 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.014 | 0.021 | 0.081 |
| 10 fF | 0.073 | 0.073 | 0.174 |
| 100 fF | 0.642 | 0.642 | 0.648 |

- Propagation delay time tables: (unlike textbook definitions that we used for our assignments, here we will use 50% of input to 50% of output to simulate propagation delay – by keeping other inputs fixed).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.014 | 0.033 | 0.085 |
| 10 fF | 0.051 | 0.075 | 0.222 |
| 100 fF | 0.399 | 0.423 | 0.658 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 0.018 | 0.034 | 0.078 |
| 10 fF | 0.065 | 0.085 | 0.216 |
| 100 fF | 0.501 | 0.521 | 0.727 |

- Static Power (cover all possible input combinations based on number of inputs).

| Condition (A) | Power (nW) |
| --- | --- |
| 0 | 0.44 |
| 1 | 0.108 |