For combinations cells (depending on number of inputs – example, inverter has only A input), the following characterizations have to be performed and filled. Remove all unwanted rows.

- Input pin capacitances:

| Input Pins | Rise Cap (fF) | Fall Cap (fF) | Average Cap (fF) |
| --- | --- | --- | --- |
| A | 3.86 | 3.95 | 3.91 |

- Transition Time Table: (please strictly consider 20% and 80% of VDD for transition time)

(i) Output Rise Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 25.89 | 28.32 | 26.49 |
| 10 fF | 72.04 | 72.22 | 76.19 |
| 100 fF | 515.16 | 515.17 | 643.66 |

(ii) Output Fall Transitions (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 41.11 | 40.46 | 108.65 |
| 10 fF | 86.66 | 86.36 | 170.17 |
| 100 fF | 510.58 | 510.60 | 521.65 |

- Propagation delay time tables: (unlike textbook definitions that we used for our assignments, here we will use 50% of input to 50% of output to simulate propagation delay – by keeping other inputs fixed).

(i) Cell Rise Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 1046.98 | 1156.46 | 1535.3 |
| 10 fF | 1080.9 | 1191.40 | 1569.7 |
| 100 fF | 1366.55 | 1485.6 | 1533.88 |

(ii) Cell Fall Delay (in ns) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 50.31 | 66.27 | 168.22 |
| 10 fF | 88.4 | 105.03 | 242.13 |
| 100 fF | 413.77 | 431.06 | 365.87 |

- Static Power (cover all possible input combinations based on number of inputs).

| Condition (A) | Power (nW) |
| --- | --- |
| 0 | 131 |
| 1 | 1.72 |

- Dynamic Power Table:

(i) Rise Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 12171 | 11763 | 12414 |
| 10 fF | 27749 | 27439 | 27504 |
| 100 fF | 164833 | 163974 | 153631 |

(ii) Fall Power (in nW) [Input slew vs output capacitance].

Related pin A: (i.e., other input pins are held constant)

|  | 10 ps | 100 ps | 1000 ps |
| --- | --- | --- | --- |
| 0.5 fF | 2800 | 2050 | 2472 |
| 10 fF | 2856 | 2268 | 2469 |
| 100 fF | 1868 | 1485 | 1099 |